醫用超音波原理

Computer Homework #2: Beam Formation

Due 12:00am 11/29/2000 by emailing to

paichi@cc.ee.ntu.edu.tw

- Load hw2_dat.mat. In this data file, apertureU defines a uniform aperture and apertureH defines an aperture with non-uniform weighting. The spacing between two points in both cases is defined by d0 in mm. The vector pulseF defines a pulse spectrum of a particular excitation with the frequency axis specified by faxis (in MHz). Finally, the sound propagation velocity is defined by soundV in mm/usec. In all figures, please label all axes.

- Assuming a continuous wave at 5MHz from the far field and zero incidence angle, plot the magnitude of the one-way diffraction patterns for both apertureU and apertureH (in dB). The horizontal axis should be sin from –1 to 1. (20%)

- Assuming a pulse wave which has the frequency response specified by pulseF and faxis, plot the magnitude of the one-way, far-field diffraction patterns (zero incidence angle) for both apertureU and apertureH (in dB). The horizontal axis should be sin from –1 to 1. (20%)

- Calculate the –6dB, -10dB and –20dB mainlobe widths of the diffraction patterns obtained from 1 and 2. Comment on your answers and the sidelobe levels between two different apodization schemes. (20%)

- Repeat 3 if the incidence angle is at 45 degrees. Justify your answers (20%)

- Repeat 1 if every 16th element of the apertures (i.e., 16, 32, 48,…) is set to zero. Justify the answers with the expected grating lobe locations described in class notes. (20%)

- (Bonus) Use the simulation programs to explore and justify issues relevant to this topic.