醫用超音波原理

Computer Homework #3: Doppler Processing

Due 12:00am 1/3/2001 by emailing to

paichi@cc.ee.ntu.edu.tw

- Load hw3_dat.mat. In this data file, DopplerData represents a PW Doppler data set. The size of the data is 128 X 400, representing gated received data with 128 firings and 400 data samples for each firing. In the same data file, the following parameters are also included:

  - PRI: pulse repetition interval in usec.

  - fs: sampling frequency in MHz.

  - f0: center frequency of the transmitted pulse in MHz.

demodulation

low pass filtering

summing

For each firing, perform the following processing. The total number of samples will reduce from 400 to a single complex sample after the processing. Repeat the processing for all firings in order to obtain 128 complex samples.

- Based on the 128 complex samples, calculate the velocity profile using FFT. The profile should be plotted on a logarithmic scale with a 40dB range. The horizontal axis should also be labeled with proper velocity scale. (40%)

- Ignore all even samples, re-do Problem 1 by using only the odd samples. Explain your results. (20%)

- Based on the original complex samples, calculate and plot the mean velocity, variance and energy as the number of samples increases from 2 to 128. Note that the discrete correlation function is defined as the following (40%)

R(n⋅T)=1N−ni=1N−nS((i+n)⋅T)S(i⋅T).

- (bonus) Explore any related Doppler issues based on the dataset.