醫用超音波原理

Computer Homework #1: Speckle Statistics

Due 5:00pm 4/3/2007 by emailing to

paichi@cc.ee.ntu.edu.tw

1.	Create an array of 10,000 complex data with the following statistics:

- Uniform distribution of amplitude in [0, 1].

- Uniform distribution of phase in [0, 2].

Plot the histograms of the amplitude and intensity of the above data.

- Create a new array of N data points based on the original array (N=10,000, 5,000, 2,000, 1,000 and 500). The ith point of the new array is the sum of M consecutive data points (M=1, 2, 5, 10 and 20) of the original array (from (i-1)*M+1 to i*M). Calculate and plot the ratio of the mean to the standard deviation of the amplitude and intensity arrays as a function of M.

- Repeat 1 and 2 by making the amplitude distribution normal with (0, 1).

- Repeat 1 and 2 by making the phase distribution normal with (0, 1).

- Repeat 2 by using amplitude of the data obtained from 1. Discuss the results with the results from 2.

- (bonus, not required) Use the program to investigate any issues relevant to this topic (speckle statistics).